Report

Image processing of the microscope photo

I have done this work in python, with the libraries numpy and cv2.

Objective: extract the period of the crystal striations.

Workflow

- First, I loaded the image into the program

Figure 1 Original image

- I applied on the image a Sobel x-axis kernel to emphasis the x-edges. Then, I rotated (manually) the image, such that the striations lines will be parallel to the image borders.

Because of the rotation, I also had to resize the image to get the largest possible area crop window within the rotated image (see explanation images below).

Figure 2 Filtered and rotated image

- I summed each line of the image to get a unique line of pixel, which represent the y-average of the image. You can see below the summed image (I duplicated the line on the y-axis).

Figure 3 Summed image

- Now we can perform a Fourier transform on this line. We get the next graph:

Figure 4 Frequency decomposition of the summed image

Analysis

We can read a maximum peak at f=0.07 Hz. This value corresponds to the pixels, not to the real distance. Then we need to convert it.

According to the microscope, the scale is

10.0678px→1μm

The period we computed is d=10.07≈14.3 px

Then the actual period is

14.3∙0.0678=0.969 μm